SOURCE CODE

IPO CHART

| INPUT | PROCESS | OUTPUT |
| --- | --- | --- |
| Physical E-Stop Relay • Digital Input (GPIO 23) • Reads High/Low state for safety interrupt line. | 1. Safety Interrupt Evaluation • Check GPIO 23 state. • If HIGH, disarm motor immediately and enter lockout state. | Emergency Disarm Signal • PWM Output: 1500 μs (Neutral) to 60A ESC. • Servo Angle: Set to 90∘ (Center). |
| TF-Luna LiDAR Sensor • Serial/UART Input (UART1 @ 115,200 baud) • Distance measurement ahead in centimetres (cm). | 2. Obstacle Detection & Braking • Parse UART distance packet. • Evaluate if Distance<30 cm. • If TRUE, trigger dynamic braking override (100 ms pause). | Emergency Brake Command • PWM Output: 1500 μs (Neutral) to 60A ESC. • Throttle held at 0%. |
| Left Sharp IR Sensor • Analog Input (GPIO 34) • ADC value (0-4095) converted to distance (cm). | 3. Track Centering Error Calculation • Calculate lateral differential error: Error=Left Distance-Right Distance | Error Value (e) • Positive = Drifted right (steer left). • Negative = Drifted left (steer right). |
| Right Sharp IR Sensor • Analog Input (GPIO 35) • ADC value (0-4095) converted to distance (cm). | 4. PID Closed-Loop Steering Control • Proportional: P=Kp⋅e • Integral: I=Ki⋅e dt (Clamped: -50 to 50) • Derivative: D=Kd⋅dedt • Total Adjustment =P+I+D | Raw Target Servo Angle Angle=90∘+Adjustment |
| Hardware & Mechanical Limits • Servo Min Angle: 55∘ • Servo Max Angle: 125∘ • Target Loop Frequency: 250 Hz (4 ms) | 5. Output Signal Constraining & Timing • Clamp steering angle between 55∘ and 125∘. • Execute 4 ms delay to maintain strict 250 Hz execution rate. | Physical Actuation Outputs • DS3218 Servo (GPIO 18): PWM angle (55∘-125∘). • 60A ESC (GPIO 19): PWM signal (1580 μs drive pulse). |

FLOW CHART

ARDUINO IDE CODE

#include <Arduino.h>

#include <ESP32Servo.h>

// ============================================================================

// 1. HARDWARE PIN DEFINITIONS & CONSTANTS

// ============================================================================

#define SERVO_PIN        18   // DS3218MG Digital Steering Servo

#define ESC_PIN          19   // 60A Brushless ESC Motor Driver

#define ESTOP_RELAY_PIN  23   // Latching E-Stop Hardware Input

#define LEFT_IR_PIN      34   // Left Sharp GP2Y0A21YK0F Analog IR

#define RIGHT_IR_PIN     35   // Right Sharp GP2Y0A21YK0F Analog IR

#define TF_LUNA_RX       16   // ESP32 RX2 connected to TF-Luna TX

#define TF_LUNA_TX       17   // ESP32 TX2 connected to TF-Luna RX

// Actuator Bounds & Calibrations

#define SERVO_CENTER_DEG 90

#define SERVO_MIN_DEG    55   // Clamped Maximum Left Turn Angle

#define SERVO_MAX_DEG    125  // Clamped Maximum Right Turn Angle

#define ESC_NEUTRAL_US   1500 // Neutral / Stop Signal (microseconds)

#define ESC_FORWARD_US   1580 // Target Forward Drive Speed (microseconds)

// Timing Control (250 Hz = 4 ms per loop)

#define TARGET_LOOP_HZ   250

const unsigned long LOOP_TIME_US = 1000000 / TARGET_LOOP_HZ;

// ============================================================================

// 2. EMBEDDED PID CONTROLLER CLASS

// ============================================================================

class PIDController {

private:

float kp, ki, kd;

float prevError;

float integral;

float minOutput, maxOutput;

public:

PIDController(float p, float i, float d, float minOut, float maxOut)

: kp(p), ki(i), kd(d), minOutput(minOut), maxOutput(maxOut), prevError(0.0f), integral(0.0f) {}

void reset() {

prevError = 0.0f;

integral = 0.0f;

}

float compute(float setpoint, float currentVal, float dt) {

float error = setpoint - currentVal;

// Proportional term

float pTerm = kp * error;

// Integral term with anti-windup clamping (-50 to 50)

integral += error * dt;

integral = constrain(integral, -50.0f, 50.0f);

float iTerm = ki * integral;

// Derivative term

float derivative = (dt > 0.0f) ? (error - prevError) / dt : 0.0f;

float dTerm = kd * derivative;

prevError = error;

// Calculate raw angle output centered around 90 degrees

float output = 90.0f + (pTerm + iTerm + dTerm);

return constrain(output, minOutput, maxOutput);

}

};

// ============================================================================

// 3. GLOBAL HARDWARE OBJECTS & STATE VARIABLES

// ============================================================================

Servo steeringServo;

Servo escMotor;

// PID Tuning Constants: Kp=1.8, Ki=0.05, Kd=0.4

PIDController pid(1.8f, 0.05f, 0.4f, SERVO_MIN_DEG, SERVO_MAX_DEG);

unsigned long lastLoopTime = 0;

// ============================================================================

// 4. HELPER FUNCTION DECLARATIONS

// ============================================================================

int readTFLunaDistance() {

HardwareSerial &lidarSerial = Serial2;

if (lidarSerial.available() >= 9) {

if (lidarSerial.read() == 0x59 && lidarSerial.read() == 0x59) { // Header bytes

uint8_t lowByte = lidarSerial.read();

uint8_t highByte = lidarSerial.read();

// Clear remaining 5 frame bytes from serial buffer

for (int i = 0; i < 5; i++) {

lidarSerial.read();

}

return (highByte << 8) | lowByte; // Distance in cm

}

}

return -1; // Return -1 if no valid frame available

}

float readAnalogDistance(int pin) {

int rawADC = analogRead(pin);

if (rawADC < 100) return 80.0f; // Default out-of-range fallback

// Convert 12-bit ADC (0-4095) at 3.3V to approximate distance in cm

float voltage = rawADC * (3.3f / 4095.0f);

float distanceCm = 27.86f * pow(voltage, -1.15f);

return constrain(distanceCm, 10.0f, 80.0f);

}

// ============================================================================

// 5. SETUP ROUTINE

// ============================================================================

void setup() {

// Start Serial Communications

Serial.begin(115200);

Serial2.begin(115200, SERIAL_8N1, TF_LUNA_RX, TF_LUNA_TX);

// Configure GPIO Pin Modes

pinMode(ESTOP_RELAY_PIN, INPUT_PULLUP);

pinMode(LEFT_IR_PIN, INPUT);

pinMode(RIGHT_IR_PIN, INPUT);

// Allocate Hardware Timers for ESP32 Servo Library

ESP32PWM::allocateTimer(0);

ESP32PWM::allocateTimer(1);

steeringServo.setPeriodHertz(50);

escMotor.setPeriodHertz(50);

steeringServo.attach(SERVO_PIN, 500, 2500);

escMotor.attach(ESC_PIN, 1000, 2000);

// Set Initial Safe States

steeringServo.write(SERVO_CENTER_DEG);

escMotor.writeMicroseconds(ESC_NEUTRAL_US);

// Delay 2 seconds for ESC arming/calibration signal to settle

delay(2000);

lastLoopTime = micros();

}

// ============================================================================

// 6. MAIN CONTROL LOOP (250 Hz Deterministic Loop)

// ============================================================================

void loop() {

unsigned long currentMicros = micros();

// Enforce strict 4ms (250 Hz) execution timing

if (currentMicros - lastLoopTime >= LOOP_TIME_US) {

float dt = (currentMicros - lastLoopTime) / 1000000.0f;

lastLoopTime = currentMicros;

// --------------------------------------------------------------------

// HARDWARE SAFETY CHECK: E-Stop State Interrupt

// --------------------------------------------------------------------

if (digitalRead(ESTOP_RELAY_PIN) == HIGH) {

escMotor.writeMicroseconds(ESC_NEUTRAL_US); // Neutral throttle

steeringServo.write(SERVO_CENTER_DEG);       // Straight steering

pid.reset();

return; // Halt further execution

}

// --------------------------------------------------------------------

// STEP 1: SEE (PERCEPTION)

// --------------------------------------------------------------------

int frontDistanceCm = readTFLunaDistance();

float leftDistCm = readAnalogDistance(LEFT_IR_PIN);

float rightDistCm = readAnalogDistance(RIGHT_IR_PIN);

// --------------------------------------------------------------------

// SAFETY OVERRIDE: Emergency Brake (<30 cm obstacle alert)

// --------------------------------------------------------------------

if (frontDistanceCm > 0 && frontDistanceCm < 30) {

escMotor.writeMicroseconds(ESC_NEUTRAL_US);

steeringServo.write(SERVO_CENTER_DEG);

delay(100);

return;

}

// --------------------------------------------------------------------

// STEP 2: THINK (CONTROL - PID Wall Centering)

// --------------------------------------------------------------------

// Error = Left Distance - Right Distance. Setpoint = 0.0 (Equal spacing)

float currentError = leftDistCm - rightDistCm;

float targetAngle = pid.compute(0.0f, currentError, dt);

// --------------------------------------------------------------------

// STEP 3: ACT (ACTUATION)

// --------------------------------------------------------------------

steeringServo.write((int)targetAngle);

escMotor.writeMicroseconds(ESC_FORWARD_US);

}

}